Class 4 OH&S

Identify hazards

Implement controls in accordance with OHS policies and procedure e.g. Manual handling, Near miss reports, Hazard alert, Injury report, Duties of employers/employees

Will require training, resources, equipment and clothing.

Communications e.g. radio, mobile phone

First aid Training and equipment

Plant and equipment maintained to manufacturers specifications

Rotate repetitive tasks where possible

Insurance, workcover

| Hazard | Control Measure |
| --- | --- |
| dust | Water, mask |
| noise | PPE - Ear protection regulate exposure |
| solar radiation | PPE – clothing. Sunscreen, shade |
| air and soil borne micro-organisms | Water, mask |
| chemicals and hazardous substances Handling Storage/transport | PPE, directions for use MSDS, procedures. Training e.g. ChemCert Hazardous substances stored and labelled correctly Provide mechanical ventilation All personnel provided with appropriate PPE |
| sharp hand tools and equipment | PPE, directions for use, procedures. Training |
| Manual Handling Handling of equipment Use of heavy hand held tools e.g brushcutter Handling of heavy objects | Procedures, Use of lifting aids Imposed restrictions on certain activities Requirements for two person lifts Training of employees Use of support harness Limits on duration of use Provide mechanical aids Redesign object or task |
| disturbance or interruption of services | Provide alternate services |
| slippery and uneven surfaces | Footwear Care Access routes kept clear |

| Biological Hazards e.g. infectious diseases Bites, Stings, | Provide appropriate waste disposal containers Provide employees with PPE Develop safe working procedures and staff training Immunisation program Implement microbiological control procedures First aid training/resources |
| --- | --- |
| machinery and machinery parts | Procedures, training, care, guards |
| moving vehicles Trucks entering, existing a work site Working in close proximity to roads | Procedures, training, care, signage, visual/auditory cues Use of traffic signal men Installation of temporary traffic signals Use of Safety Signs Speed restriction signs displayed and enforced Use of witches hats or temporary barriers to cordon off sections of road Closure of road Use of Safety Signs Speed restriction signs displayed and enforced |